| 2004 pins | colors | Names | colors | 2006 pins |
| --- | --- | --- | --- | --- |
| 1,4 | B | GND,Mode | B | 1,21 |
| To audio GND |  |  |  |  |
| 2,3 | Y | +B,BAT | Y | 2 |
| From audio 6 |  |  |  |  |
| 6 | LG | ACC | LG | 5 |
| From audio 10 |  |  |  |  |
| 8 | W/G | SPEED | W/G | 66 |
| 9 | R/L | ILL | R/L | 61 |
| From audio 9 |  |  |  |  |
| 11 | OR | RV | OR | 65 |
| To the one “PU”, near the shift, if not found, connect to navi 27 |  |  |  |  |
| 12 | L/W | Voice + | L/W | 12 |
| 13 | W/B | Voice - | W/B | 14 |
| 14 | L | YS | L | 50 |
| 15 | R | B | R | 46 |
| ????? |  | RGB GND |  | 47 |
| 17 | G | RGB GND, SYNC GND | G | 49 |
| 18 | B | R | B | 44 |
| 20 | P | RGB SYNC | P | 48 |
| 21 | W | G | W | 45 |
| 26 | SB | IGN SIG |  | To wire from the audio which is ING-SIG, color light green. |
| 27 | Y/G | IGN | Y/G | 63 |
| From Metter Control 41,42??? |  |  |  |  |
| 31 | B | GND, BUS Shield |  | 68(1) |
| 33 | G | SW-NAVI LIMIT SW | R/W | 22 |
| 35 | B | NAV-DISP,BUS + | W | 69 |
| 37 | R/B | RLY+, Guide On | R/B | 24 |
| 40 | GND | RLY-, Shield | B | 13 |
| 49 | B/R | BUS+ | LG | 71 |
| 50 | W/R | BUS- | Pu | 72 |
| 51 |  | Shield, Bus Shield,GND |  | 68(2) |
| ???? |  | Bus- |  | 70 |
| 56 |  | Antenna |  | 73,74 |